### How to use the  Samples  and  Manual  MC  Override  features.

- 1.	Select the desired kiln number from the Overall Kiln Status (OKS).

- 2.	When starting a kiln check the “Use manual MC Override” check box to run MC schedules based on your samples. Specify “Starting Avg. MC” (starting point in MC schedule) & “MC Loss/Day” (schedule progress rate).

- 3.	In the Individual Kiln Status Screen (IKS), Click “Samples” button in to enter Samples page.

- In the samples page, set up your first sample by entering the individual sample information, then click the “add” button, in the top right corner under “Add/Edit Samples”.

Wet sample wafer wt.

Oven Dry Sample wafer wt.

- 5.	First samples information is added as seen below.

6.	Keep in mind that you can navigate away from the page without losing your sample data. The “Clear Table” button will clear the whole table when desired for the next charge (password required). You can also hover your mouse over the sample number label at the top of the column (“Sample 1” in the pic) to edit or delete a sample column at any time.

- 7.	Example below of a second samples information added. Repeat step 4 for each sample. (unlimited sample quantity)

- 8.	With a sample scale connected to your network or MP32, click “Insert from scale” button in the samples designated column to enter sample weight. If using a separate scale, type the sample weight in the data box then click the “add” button.

- 9.	Below, the first sample weight for Sample 1 has been added.

- You will see the calculated MC% and the option to use this sample in your Avg. MC%. Note loss/day is not calculated yet.

- 10. Sample #2 added.

- 11.	If you are at this point with a running kiln, verify you're running a MC based schedule (DM) and “Use Avg MC Override” is enabled in the “Change Setup” page (2nd pic below) by clicking “Change Setup” button.

12.	If starting a kiln using sample MC% data. Make sure the schedule you choose from your library is an MC based schedule, and the “Use Avg MC Override” box is checked in the “Start Kiln” page. “Starting Avg. MC” and “MC Loss/ Day” can be left blank at this point as the schedule starts in heating up and warming thru segments. MC% data is not required for these segments.

- Once the kiln is started, click the “Samples” button from the IKS screen. This will take you back to your samples setup page from step #10.

14.	Use desired sample MC's in Avg MC by checking the box, then click “Set Avg MC” button.

- 15.	You will then be prompted to enter a “Loss/Day value.

- This determines schedule progression speed.

- Navigate back to the IKS screen and you will see the “Manual Avg” MC% equals your samples average.  Your MC based schedule is now advancing at your desired  loss/day  from this  MC%

- To continue adding sample weights, navigate back to the “Samples” page from the IKS page.

- 18.	Place sample on scale and use the insert from scale button or add button as described on instruction number 8.

- 19.	Each time you add a weight with the “Add” or “Insert from Scale” button it adds a row, one at a time for the respective sample column. The option to “Set Avg MC” is always available in the last row of column segments.

- As you hover your mouse over a row of sample weights/values, the row expands to give you more information and options for the row, and collapses as your mouse leaves the row.

21.	The picture below has an expanded last row showing options and added info.

- 22. If you have multiple sample scales connected to your network, select the correct sample scale from the drop down menu before you start inserting sample weights using the “Insert from Scale” button.

For further questions on how to use the Samples feature please contact Lignomat.